Architecture review 4-12-21

Menu Wall

Reduce wall from 8’6” to 8’4” by removing 2” from the south side of the wall, we have 8ft 3.5” of equipment going on back of this wall and want to open the south view to the make line as much as possible

Change equipment on cut table side of this wall from left to right (south to north):  24” dip cup refrigerator (new item number), 54”L x 30”W cut table (item 25), 21 1/2” x 72” wide heated shelf (item 51)

- Wire heated shelf should go up against the cut table ss full length shelf that is 18” wide (item 53)

- Wire shelf will likely have a gap off the back of the hood wall.  Is there an attractive solution to fill this void or is it necessary?

- The wire warming shelf will only extend past the hood wall approximately 12”.

Menu wall / hood wall equipment

Move equipment to the left (toward the menu wall) approx 15”

Soffit detail

Extend glass by 1 ft from 6ft to 7ft (reduce the sheetrock portion of the soffit) , but keep the 3 lights positioned as shown, typically the lights would all be within soffit area and not in the glass area…..keep it balanced and proportional.  We will defer to Architect’s “eye” on this but thought we should try and maximize the glass as much as possible.

Counter

Counter – Sheet A711, plan view of counter “orders counter plan” no end wing wall shown with dashed line supports.

Front of house tile:

Single pattern of tile not shown. On sheet A901

Menu holder (adjacent wall to counter (outer office wall) is not shown (along the office wall)

See proto details page:  Figure 5 Page A1.4 (side panel section)

Food Equipment schedule and changes:

- Remove item 9 slicer from delivery station

- Relabel item 49 from processor to Mixer

- Item 27 is a reinforced 5ft table needs to be added to equipment list (it is listed in prototype)

- Change item 41 to full stainless shelving (all shelves)

- Item 56 is not on the equipment list, Make this a 4ft table with can opener

- Place another table (item 26) below item 39 shelf on back side of hood wall, extend for remaining length of the wall round to nearest foot.

- Item 39 shelf change to full stainless steel

- There should be two (2) item 2 infrared heaters on shelf 51 (top and middle shelf)

- Remove wire shelf 40 on menu wall and extend full ss shelf item 53 to 8ft centered on back of menu wall

  - POS monitor to be mounted under the shelf (off the table – mounted to wall)

- Missing dip cup refrigerator at end of line on menu wall (2ft)

- Reduce Item Item 25 to 54” long cut table length.

- Change Shelf in walk-in cooler item 37 to a single 3ft long shelf to fit inside a 6ft walk-in cooler (ID)

  - FED TO ENSURE SHELVIING WILL FIT INSIDE WALKIN COOLER.

- Make line refrigeration Jon and Dan to get alternate mfg for price comparison and value comparison from specified True unit (SLW store as example)